Echo Vue by Illy Robotics

Production Architecture Topology & Validation

Generated: April 2026  |  Document Type: Architecture Reference
Confidential — Illy Robotics Internal

# Table of Contents

1. Architecture Validation Summary

2. Production Architecture — 3-Layer Topology

2.1 Layer 1: Edge (On-Premise Per Environment)

2.2 Layer 2: Frontend (User Devices)

2.3 Layer 3: Cloud Backend — AI MBL Engine

3. Multi-Tenant Isolation — Network Binding

3.1 Hospital A vs Hospital B Scenario

3.2 Isolation Guarantees

4. Data Flow — Complete Lifecycle

4.1 Setup & Calibration

4.2 Live Presence Detection

5. Cloud Backend Internal Architecture

6. Security Architecture

7. Gaps: Current Codebase vs Production Vision

8. USPTO Drawing Corrections

# 1. Architecture Validation Summary

The Gemini-generated USPTO drawing is architecturally misleading. It depicts the "AI Brain" as a local/on-premise component adjacent to the ESP32 and cameras, with no cloud backend visible. This implies all heavy AI/ML processing happens locally, which contradicts the production vision and would not scale commercially.

## USPTO Drawing Issues vs Correct Production Vision

| Issue in USPTO Drawing | Correct Production Vision |
| --- | --- |
| "AI Brain" shown as local/on-premise box | AI MBL Engine runs in the CLOUD — GPU-intensive compute |
| No cloud backend depicted | Cloud is the central nervous system of Echo Vue |
| No multi-tenant isolation shown | Each user/org gets isolated data pipelines |
| Camera/ESP32 seem to do processing | ESP32 = dumb sensor, Camera = calibration-only then OFF |
| No network binding concept | Environments are bound to network IPs — can't manage remotely |
| Monolithic single-user flow | Multi-tenant SaaS serving homes, hospitals, businesses simultaneously |

# 2. Production Architecture — 3-Layer Topology

Echo Vue operates across three distinct layers. No AI/ML computation occurs on the edge (ESP32) or frontenddevices. ALL heavy processing is centralized in the cloud AI MBL Engine.

## 2.1 Layer 1: Edge (On-Premise Per Environment)

┌──────────────────────────────────────────────────────────┐
│  ON-PREMISE (Hospital Floor / Home / Business)           │
│                                                          │
│  ┌──────────┐    WiFi CSI     ┌────────────────────┐    │
│  │ WiFi 6   │ ──────────────► │ ESP32-S3           │    │
│  │ Router   │   Subcarrier    │ Bridge Node        │    │
│  │          │   Data          │                    │    │
│  └──────────┘                 │ • CSI Extraction   │    │
│                               │ • BLE Scanning     │    │
│  ┌──────────┐    BLE RSSI     │ • TLS 1.3 Stream   │    │
│  │ BLE Tags │ ──────────────► │ • NO AI/ML         │    │
│  │ (Staff,  │                 └────────┬───────────┘    │
│  │ Patient) │                          │ TLS 1.3        │
│  └──────────┘                          ▼                │
│                                ═══ TO CLOUD ═══         │
└──────────────────────────────────────────────────────────┘

### ESP32 Responsibilities

| ESP32 Does NOT | ESP32 ONLY Does |
| --- | --- |
| Run any ML models | Extracts raw CSI (242 subcarriers × 2×2 MIMO × 100Hz) |
| Render any images | Passively scans BLE advertisements (RSSI + MAC) |
| Store persistent data | Streams both over TLS 1.3 to cloud |
| Make decisions about presence | Reports LED status ring for local feedback |

## 2.2 Layer 2: Frontend (User Devices — Same Network Required)

┌──────────────────────────────────────────────────────────┐
│  USER DEVICES (Must be on same network as environment)   │
│                                                          │
│  ┌────────────────────────────────────────────────────┐  │
│  │  Echo Vue Web Portal (Next.js / React)              │  │
│  │                                                      │  │
│  │  • Environment Setup & Floor Plan Editor             │  │
│  │  • Calibration Wizard (temp camera for onboarding)   │  │
│  │  • Live 3D Floor Plan Viewer                         │  │
│  │  • Vitals Dashboard                                  │  │
│  │  • Staff/Patient Presence Overlay                    │  │
│  │                                                      │  │
│  │  DOES NOT:                                           │  │
│  │  ✗ Run AI models in production                       │  │
│  │  ✗ Process CSI data                                  │  │
│  │  ✗ Compute skeletal poses                            │  │
│  │  ✗ Store other tenants' data                         │  │
│  │                                                      │  │
│  │  ONLY:                                               │  │
│  │  ✓ Renders pre-computed vectors from cloud           │  │
│  │  ✓ Displays 3D floor plans with avatars              │  │
│  │  ✓ Shows vitals and alerts                           │  │
│  │  ✓ Sends calibration data during setup               │  │
│  └────────────────────────────────────────────────────┘  │
└──────────────────────────────────────────────────────────┘

## 2.3 Layer 3: Cloud Backend — AI MBL Engine (The Heavy Lifter)

╔═══════════════════════════════════════════════════════════════════╗
║  ☁  ECHO VUE CLOUD — AI MBL ENGINE                              ║
║                                                                   ║
║  ┌──────────────────────────────────────────────────────────┐    ║
║  │  API GATEWAY                                              │    ║
║  │  • TLS Termination          • Rate Limiting               │    ║
║  │  • JWT Validation           • DDoS Protection             │    ║
║  │  • Network Origin Check     • Subscription Tier Enforce   │    ║
║  └──────────────────────────────────────────────────────────┘    ║
║                              │                                    ║
║  ┌──────────────────────────┴──────────────────────────────┐    ║
║  │  TENANT ISOLATION LAYER                                   │    ║
║  │  • User UUID scoping on ALL queries                       │    ║
║  │  • Environment ownership verification                     │    ║
║  │  • Network IP/ID binding (env ↔ network)                  │    ║
║  │  • Subscription tier enforcement                          │    ║
║  └──────────────────────────┬──────────────────────────────┘    ║
║                              │                                    ║
║  ┌──────────────────────────┴──────────────────────────────┐    ║
║  │  AI MBL COMPUTE ENGINE (GPU + CPU Workers)                │    ║
║  │                                                            │    ║
║  │  GPU Workloads:                                            │    ║
║  │  ├─ LatentCSI Encoder (CSI → 512-dim vectors)             │    ║
║  │  ├─ CalibrationGAN (adversarial training, 500 epochs)     │    ║
║  │  └─ Pose Inference (3D skeletal reconstruction)           │    ║
║  │                                                            │    ║
║  │  CPU Workloads:                                            │    ║
║  │  ├─ CSI Filtering (bandpass, Hampel outlier removal)      │    ║
║  │  ├─ Kalman Tracking (6-state filter, multi-person)        │    ║
║  │  ├─ RF Signature Engine (gait + breath + mass features)   │    ║
║  │  └─ Vitals Processor (HR, breathing, fall detection)      │    ║
║  └──────────────────────────┬──────────────────────────────┘    ║
║                              │                                    ║
║  ┌──────────────────────────┴──────────────────────────────┐    ║
║  │  DATA LAYER                                               │    ║
║  │  ├─ PostgreSQL (users, environments, activity logs)       │    ║
║  │  ├─ Milvus Vector DB (RF embeddings, partitioned)         │    ║
║  │  ├─ Redis (session state, real-time pubsub, cache)        │    ║
║  │  └─ Object Storage (floor plans, snapshots, heatmaps)    │    ║
║  └──────────────────────────────────────────────────────────┘    ║
╚═══════════════════════════════════════════════════════════════════╝

# 3. Multi-Tenant Isolation — Network Binding

## 3.1 Hospital A vs Hospital B Scenario

Two competing hospitals using Echo Vue will NEVER see each other's data. The AI MBL Engine processes data from multiple tenants simultaneously but returns results ONLY to the requesting tenant's authenticated session and verified network origin.

Hospital A (IP: 203.0.113.10)           Hospital B (IP: 198.51.100.20)
┌─────────────────────────┐            ┌─────────────────────────┐
│ 12 ESP32 nodes (4/floor)│            │ 8 ESP32 nodes (2/floor) │
│ Admin: admin@hosp-a.com │            │ Admin: admin@hosp-b.com │
│ Env bound: 203.0.113.10 │            │ Env bound: 198.51.100.20│
└────────────┬────────────┘            └────────────┬────────────┘
             │                                       │
             └──────────────┬────────────────────────┘
                            │
                  ┌─────────▼─────────┐
                  │  ☁ AI MBL Engine   │
                  │                    │
                  │  ┌──────────────┐  │
                  │  │Tenant Router │  │
                  │  │JWT + Network │  │
                  │  │Verification  │  │
                  │  └──────┬───────┘  │
                  │         │          │
                  │    ┌────┴────┐     │
                  │    ▼         ▼     │
                  │ Pipeline A  Pipeline B
                  │ (Hosp A     (Hosp B
                  │  ONLY)       ONLY) │
                  │                    │
                  │ DB: Row-Level      │
                  │ Security per       │
                  │ user_id            │
                  └────────────────────┘

### Network Binding — Blocked Scenario

| Scenario | Network Check | Result |
| --- | --- | --- |
| Hospital A admin ON-SITE (IP: 203.0.113.10) | 203.0.113.10 = 203.0.113.10 ✓ | ACCESS GRANTED Returns Hospital A data ONLY |
| Hospital A admin at HOME (IP: 72.14.200.5) | 72.14.200.5 ≠ 203.0.113.10 ✗ | ACCESS DENIED Must be on environment's network |
| Hospital B admin ON-SITE (IP: 198.51.100.20) | 198.51.100.20 = 198.51.100.20 ✓ | ACCESS GRANTED Returns Hospital B data ONLY Hospital A data NEVER visible |

## 3.2 Isolation Guarantees

Authentication: Google OAuth → JWT with user UUID

Authorization: Every DB query scoped to WHERE user_id = {jwt.sub}

Network Binding: Environment creation records network_id (public IP); presence scans verify request origin matches

Vector DB Isolation: Milvus collections partitioned by environment_id — RF signatures physically separate per tenant

Subscription Enforcement: Tier limits (Personal: 10 envs, Pro: 50 envs) enforced at API gateway

# 4. Data Flow — Complete Lifecycle

## 4.1 Setup & Calibration

SETUP:
  User creates account  ─►  Google OAuth  ─►  JWT issued
  User creates Environment  ─►  binds to current network IP
  User registers ESP32 node  ─►  binds to environment

CALIBRATION (one-time per person):
  1. Camera ON briefly (privacy-preserving)
  2. Frontend captures MoveNet keypoints (17-point pose)
  3. ESP32 streams CSI simultaneously
  4. Both sent to Cloud via WebSocket
  5. Cloud: LatentCSI encodes CSI → 512-dim vectors
  6. Cloud: CalibrationGAN trains (CSI ↔ pose mapping)
  7. Cloud: RF Signature extracted (gait + breathing + mass)
  8. Cloud: Embeddings stored in Milvus (env-partitioned)
  9. Camera OFF permanently → RF-only mode

## 4.2 Live Presence Detection (Camera-Free)

LIVE PRESENCE (continuous, camera-free):

  ESP32 Bridge          Cloud AI MBL Engine         Echo Vue Frontend
  ─────────────         ──────────────────          ─────────────────
       │                        │                          │
       │──CSI @ 100Hz──────────►│                          │
       │   (TLS 1.3)            │                          │
       │                        │─ Filter CSI              │
       │                        │─ LatentCSI decode        │
       │                        │─ Kalman track            │
       │                        │─ Extract vitals          │
       │                        │─ Detect falls            │
       │                        │                          │
       │                        │──Tracking Snapshot──────►│
       │                        │  (3D vectors, vitals,    │
       │                        │   alerts, positions)     │
       │                        │                          │─ Render 3D
       │                        │                          │  floor plan
       │                        │                          │─ Display
       │                        │                          │  avatars
       │                        │                          │─ Show vitals
       │                        │                          │
       │          FRONTEND DOES ZERO AI PROCESSING         │

# 5. Cloud Backend Internal Architecture

┌═══════════════════════════════════════════════════════════════════════┐
║  CLOUD INFRASTRUCTURE (Production Deployment)                        ║
║                                                                       ║
║  ┌───────────────────────────────────────────────────────────────┐   ║
║  │  LOAD BALANCER / CDN (Cloudflare / AWS ALB)                   │   ║
║  │  TLS Termination  •  WAF  •  DDoS Shield  •  GeoDNS          │   ║
║  └────────────────────────────┬──────────────────────────────────┘   ║
║                               │                                       ║
║  ┌────────────────────────────┴──────────────────────────────────┐   ║
║  │  API PODS (Horizontal Autoscale)                               │   ║
║  │  FastAPI Pod 1  │  FastAPI Pod 2  │  FastAPI Pod N...          │   ║
║  └────────────────────────────┬──────────────────────────────────┘   ║
║                               │                                       ║
║  ┌────────────────────────────┴──────────────────────────────────┐   ║
║  │  MESSAGE QUEUE (Kafka / RabbitMQ)                              │   ║
║  │  CSI Frame Ingestion  •  Async Job Distribution                │   ║
║  └──────┬─────────────────────────────────────────┬──────────────┘   ║
║         │                                          │                  ║
║  ┌──────▼──────────────┐   ┌──────────────────────▼──────────────┐  ║
║  │  GPU COMPUTE PODS    │   │  CPU COMPUTE PODS                   │  ║
║  │                      │   │                                      │  ║
║  │  LatentCSI Encoder   │   │  Kalman Tracker (stateful/env)      │  ║
║  │  (NVIDIA T4/A100)    │   │  CSI Filter Pipeline                │  ║
║  │                      │   │  RF Signature Engine                 │  ║
║  │  GAN Training Worker │   │  Vitals Processor                   │  ║
║  │  (NVIDIA A100)       │   │  (HR, Breathing, Falls)             │  ║
║  │                      │   │                                      │  ║
║  │  Pose Inference      │   │                                      │  ║
║  │  (NVIDIA T4)         │   │                                      │  ║
║  └──────────┬───────────┘   └───────────────────────┬────────────┘  ║
║             │                                        │                ║
║  ┌──────────┴────────────────────────────────────────┴────────────┐  ║
║  │  MANAGED DATA SERVICES                                          │  ║
║  │                                                                  │  ║
║  │  PostgreSQL (RDS)     │  Milvus Cluster      │  Redis Cluster   │  ║
║  │  Users, Environments  │  RF Embeddings       │  Session State   │  ║
║  │  Activity Logs        │  Partitioned/env_id  │  PubSub, Cache   │  ║
║  │  Row-Level Security   │  COSINE similarity   │  Tracking State  │  ║
║  │                       │                      │                   │  ║
║  │  Object Storage (S3/GCS)                                        │  ║
║  │  Floor Plans  •  Snapshots  •  Calibration Artifacts            │  ║
║  └──────────────────────────────────────────────────────────────────┘  ║
║                                                                       ║
║  ┌──────────────────────────────────────────────────────────────────┐ ║
║  │  OBSERVABILITY: Prometheus + Grafana + Sentry                     │ ║
║  └──────────────────────────────────────────────────────────────────┘ ║
╚═══════════════════════════════════════════════════════════════════════╝

# 6. Security Architecture

| Layer | Mechanism | Details |
| --- | --- | --- |
| Transport | TLS 1.3 | All ESP32 → Cloud and Frontend → Cloud connections encrypted |
| Authentication | Google OAuth 2.0 + JWT | 24h expiry, HS256 signed, user UUID embedded |
| Authorization | Row-Level Security | Every DB query scoped to authenticated user_id |
| Network Binding | IP Origin Verification | Environment locked to creation network; presence scans verify match |
| Tenant Isolation | UUID + Environment Partitioning | Milvus partitioned by env_id; PostgreSQL filtered by user_id |
| API Protection | Rate Limiting + WAF | DDoS protection, request validation at gateway |
| Data Separation | Logical Isolation | No cross-tenant queries possible; rendering returns only matched data |
| Camera Privacy | Automatic Shutoff | Camera used only during calibration; server-side hard stop after GAN handoff |

# 7. Gaps: Current Codebase vs Production Vision

| Component | Current State | Production Need | Priority |
| --- | --- | --- | --- |
| Network Binding | Not implemented | Environment ↔ network IP binding + verification on every request | CRITICAL |
| ESP32 TLS Connection | Protocol defined, manager is stub | Full TLS 1.3 handler with device lifecycle | CRITICAL |
| API Gateway | No rate limiting, no WAF | Rate limiting, DDoS protection, request validation | CRITICAL |
| Subscription Enforcement | Tiers defined, not enforced | Middleware check before accepting CSI streams | HIGH |
| Redis Session State | In-memory singleton | Redis-backed state for horizontal scaling | HIGH |
| Real-Time PubSub | REST polling (stale data) | WebSocket push via Redis PubSub | HIGH |
| Cloud Storage | Config exists, never used | Persist floor plans, snapshots, artifacts | HIGH |
| Monitoring | No observability stack | Prometheus + Grafana + Sentry | HIGH |
| BLE Tethering | Stub with placeholder logic | Full MAC → person matching | MEDIUM |
| Federated Learning | Flower framework stubs | Cross-env model improvement | MEDIUM |
| Edge ML Filtering | Firmware placeholder | Optional pre-filtering on ESP32 | LOW |

# 8. What the USPTO Drawing Should Show

The Gemini USPTO drawing is misleading because it depicts the system as a self-contained on-premise unit. For accurate patent drawings, the architecture should clearly show:

Three distinct zones: Edge (ESP32), Cloud (AI MBL), Frontend (User Devices) — clearly separated with network boundaries

Cloud as the processing center: ALL AI/ML computation happens in the cloud, not on-premise

Network arrows: TLS 1.3 from edge → cloud, HTTPS/WSS from frontend → cloud — showing internet traversal

Multi-tenancy: Multiple environments served by one cloud engine with tenant isolation boundaries

Network binding: Dashed line showing environment-to-network-IP binding constraint

Data return path: Cloud → frontend with "rendered vectors only" annotation — no raw data leaves the cloud

Camera lifecycle: Clearly show camera ON during calibration → OFF during live mode (server-enforced)

# Appendix: Processing Distribution by Layer

| Task | Runs On | Reason |
| --- | --- | --- |
| CSI Extraction | Edge (ESP32) | Must stream raw 242 subcarriers × 100Hz |
| BLE Scanning | Edge (ESP32) | Passive advertisement collection |
| CSI Filtering (bandpass, outlier) | Cloud Backend | Requires full history window; stateful |
| LatentCSI Encoding | Cloud Backend (GPU) | Torch model; requires GPU/VRAM |
| GAN Calibration Training | Cloud Backend (GPU) | 500-epoch training loop; requires GPU |
| Pose Inference (CSI-only) | Cloud Backend (GPU) | LatentCSI decoder; one-shot per frame |
| Kalman Tracking | Cloud Backend (CPU) | Stateful per-track; centralized multi-device state |
| RF Signature Extraction | Cloud Backend (CPU) | Gait + breathing + mass feature engineering |
| Vitals Processing | Cloud Backend (CPU) | Heart rate, breathing, fall detection |
| MoveNet Pose Detection | Frontend (calibration only) | Temporary label collection; TensorFlow.js |
| 3D Floor Plan Rendering | Frontend | Displays pre-computed vectors from cloud |
| User Settings Sync | Frontend + Cloud | Frontend debounces writes; cloud is CRUD store |